SS 300 Model Installation

The Model SS 300 Hitch Accessory is manufactured for use with all Trailer Hitch Couplers using either a 1 7/8” or 2” Trailer Hitch Coupler, whether it’s Bumper attached or attached to a Receiver adapter.

The universal kit design has 2 washers included in the SS 300 kit which can be installed either through the top of the Safety Sentry base mount plate or into the bottom of the Safety Sentry Base mount plate depending on the Trailer Hitch Coupler height requirements you determine. For covered Bumper installations, you may need to remove the Bumper cover in the area of installation so that the Trailer Ball shank protrudes sufficiently for lock washer application.

To install, determine the size of the Trailer Ball shank, either ¾” or 1” diameter. Select the Step washer that matches the shank size you are using and place through the top of the Safety Sentry Base mount plate or into the bottom of the Safety Sentry Base mount plate depending on your Trailer Hitch Coupler height requirements.

If you have a Reese Recessed Ball Hitch, you must place the Step washer into the bottom of the Safety Sentry Base mount plate in order to prevent any binding of the Trailer Ball with Safety Sentry attached.